POLICY BRIEF

Reducing Hungary's Electricity Import Dependence

A Targeted, Hour-Level Approach Based on 2022-2025 System Data

Prepared for: Government of Hungary   |   Data source: MAVIR VER hourly dataset   |   April 2026

Executive Summary

Hungary's electricity import dependence has fallen from 28.2% of load in 2022 to 20.6% in 2025. This progress is real, but the annual figure hides the actual problem: imports are highly concentrated in specific hours of the day and specific months of the year, not spread evenly across time.

Using four years of hourly MAVIR system data (2022-2025), this brief shows that the top 10% most import-intensive hours carry roughly twice the import share of normal hours. These stress hours occur overwhelmingly during winter evenings, when domestic production is structurally lower and demand is structurally higher.

The policy implication is clear: broad, untargeted measures are inefficient. Reducing import dependence further requires interventions focused on the narrow time windows where the system is actually stressed.

Our scenario modelling suggests that a combined package - 2% peak-evening demand reduction, 3% additional domestic production during winter evenings, and 100 MWh/hour of storage shifting from midday to evening - would reduce winter-evening import by approximately 13.7% and total annual import by 1.7%, with no need to change the system's overall annual structure.

1. The Problem Is Time, Not Volume

Annual trends look positive…

Hungary's net import share of total electricity load has declined for four consecutive years. This is a genuine improvement driven by rising domestic generation and more predictable planning.

…but the system is not stressed evenly

When the data is examined at the hour level, a different picture emerges. The 10% of hours with the highest net import are not random - they cluster into a predictable pattern, as shown below.

| Indicator | High-import hours (top 10%) | Normal hours | Gap |
| --- | --- | --- | --- |
| Average system load (MW) | 5,865 | 4,724 | +24% |
| Average domestic production (MW) | 3,180 | 3,661 | -13% |
| Net-import share of load | 46.1% | 21.9% | +2.1× higher |

Source: own calculation on MAVIR hourly data, 2022-2025 (N ≈ 35,000 hours).

High-import hours are not caused by a single factor. They are the combination of above-average demand and below-average domestic supply. The import share in these hours is 46% - more than double the average. This combination is most frequent during winter evenings (17:00-22:00, November-February).

2. What the Data Recommends

Because the problem is concentrated in time, the most cost-effective response is also concentrated in time. Three intervention levers were tested against the 2022-2025 dataset:

- Peak shaving - focuses on reducing consumption during winter evening peaks (17:00-22:00, Nov-Feb).

- Domestic production - adds generation in the same high-risk hours (e.g. scheduled availability of dispatchable capacity).

- Storage shifting - charges batteries or pumped-hydro during low-demand midday hours and discharges them into the evening peak.

Scenario results

| Intervention scenario | Annual import reduction | Winter-evening peak reduction |
| --- | --- | --- |
| Peak shaving: -1% winter evening load | 0.4% | 2.5% |
| Peak shaving: -5% winter evening load | 2.0% | 12.4% |
| Domestic production +3% (winter evenings) | 0.7% | 4.5% |
| Domestic production +5% (winter evenings) | 1.2% | 7.4% |
| Storage: 100 MWh/h shift midday → evening | 0.2% | 4.2% |
| Storage: 200 MWh/h shift midday → evening | 0.5% | 8.5% |
| Combined package (-2% load, +3% prod, 100 MWh/h storage) | 1.7% | 13.7% |

All scenarios applied only to winter evening hours (Nov-Feb, 17:00-22:00). Results from own simulation on 2022-2025 hourly data.

3. Recommendations

On the basis of the evidence above, we submit three concrete recommendations for consideration:

Recommendation 1 - Peak-focused demand response. Establish a demand-response programme explicitly targeting the 17:00-22:00 window in winter months, via industrial load tariffs, smart-metering time-of-use incentives, and public-sector load scheduling. A 5% reduction in this window alone would cut winter-evening import by 12%.

Recommendation 2 - Prioritise dispatchable domestic capacity for evening hours. Scheduled availability of domestic generation in the winter evening window should be treated as a distinct policy objective, not absorbed into general annual capacity targets. A 3% increase here is worth more than a 3% increase distributed across the year.

Recommendation 3 - Deploy storage against the midday-to-evening gap. The data shows consistent surplus conditions during winter midday hours (11:00-14:00). Battery and pumped storage capable of a 100-200 MWh/hour shift into the evening peak would deliver 4-9% peak-import reduction with no additional generation needed.

4. Conclusion

Hungary's remaining import dependence is not a general capacity problem - it is a timing problem. The annual averages are improving, but the stress hours remain concentrated in winter evenings. Policy that recognises this structure, rather than treating the whole year as equally problematic, can deliver measurable security-of-supply gains at a fraction of the cost of broad capacity expansion.

Assumptions and limitations

- Direct substitution - Scenario simulations assume that reductions in peak-evening load translate one-to-one into reductions in net import. In practice, system operator rebalancing may partly offset this; the estimates should therefore be read as upper bounds for the modelled levers in isolation.

- Underlying trend - The 28.2% → 20.6% improvement was driven partly by rising own production and partly by favourable market conditions in 2025. The brief does not assume this trend continues automatically.

- Data scope - The 2022-2025 window covers post-pandemic recovery, the 2022 gas-price shock, and the subsequent normalisation. Seasonal patterns identified here are robust across all four years; absolute levels may differ in future.

Proposed next steps

A three-stage rollout would allow each intervention to be measured against the hourly baseline established in this study: (I) a one-winter pilot of evening time-of-use tariffs in a defined consumer segment, (II) a reserve-capacity auction specifically for 17:00-22:00 winter availability, and (III) a targeted call for storage projects dimensioned against the midday-evening gap. After one full winter, the same hourly indicators used here could be re-evaluated to quantify actual impact.

Methodology note: Analysis based on 35,064 hourly observations from MAVIR's VER dataset (Jan 2022 - Dec 2025). High-import hours defined as the top 10% of net-import values. Scenario simulations apply interventions only to winter evening hours (Nov-Feb, 17:00-22:00) and re-compute annual and peak-window totals against baseline.